# Practical Task 1 — ESP32 Development Environment Setup with PlatformIO

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Create, build, upload, and test an ESP32 Arduino project using VS Code and PlatformIO.

# 2. Components / Software

ESP32 DevKit, USB data cable, computer with VS Code and PlatformIO.

# 3. Procedure

Install VS Code and the PlatformIO IDE extension.

Create a new PlatformIO project: Board = Espressif ESP32 Dev Module; Framework = Arduino.

Open platformio.ini and verify the ESP32 board and Arduino framework configuration.

Create/edit src/main.cpp.

Build the project, connect the ESP32 by USB, select the correct COM port, and upload.

Open Serial Monitor at 115200 baud and verify the student information and ESP32 Ready message.

Verify the onboard LED blinks at 1-second ON / 1-second OFF intervals.

# 4. Solution / Program

platformio.ini:

[env:esp32dev]
platform = espressif32
board = esp32dev
framework = arduino
monitor_speed = 115200

src/main.cpp:

#include <Arduino.h>

#ifndef LED_BUILTIN
#define LED_BUILTIN 2
#endif

void setup() {
  Serial.begin(115200);
  pinMode(LED_BUILTIN, OUTPUT);

  Serial.println("Embedded IoT Systems");
  Serial.println("Student Name: YOUR NAME");
  Serial.println("Roll Number: YOUR ROLL NO");
  Serial.println("ESP32 Ready");
}

void loop() {
  digitalWrite(LED_BUILTIN, HIGH);
  delay(1000);
  digitalWrite(LED_BUILTIN, LOW);
  delay(1000);
}

# 5. Expected Result

Serial Monitor should show Embedded IoT Systems, Student Name, Roll Number, and ESP32 Ready. The onboard LED should repeatedly turn ON for 1 second and OFF for 1 second.

# 6. Troubleshooting / Verification

If upload fails, check the COM port, USB data cable, board selection, and serial monitor port.

If the onboard LED does not blink, verify the board's built-in LED pin; many ESP32 DevKit boards use GPIO 2.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Modify the blink interval to 500 ms and then 2000 ms. Rebuild and upload after each change.