# Practical Task 2 — ESP32 Virtual Prototyping Using Wokwi

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Simulate an ESP32, pushbutton, and LED circuit before physical hardware wiring.

# 2. Components / Software

Wokwi ESP32 simulation, pushbutton, LED, 220Ω resistor, virtual ground/3.3V wiring.

# 3. Procedure

Create a Wokwi ESP32 project or open the PlatformIO project in Wokwi.

Connect the pushbutton to GPIO 18 and GND; use INPUT_PULLUP in software.

Connect an LED through a 220Ω resistor to GPIO 2 and GND.

Run the simulation.

Press the button: the LED should turn ON and Serial Monitor should report PRESSED.

Release the button: the LED should turn OFF and Serial Monitor should report RELEASED.

# 4. Solution / Program

Use the following src/main.cpp:

#include <Arduino.h>

const int BUTTON_PIN = 18;
const int LED_PIN = 2;

void setup() {
  Serial.begin(115200);
  pinMode(BUTTON_PIN, INPUT_PULLUP);
  pinMode(LED_PIN, OUTPUT);
}

void loop() {
  bool pressed = (digitalRead(BUTTON_PIN) == LOW);

  digitalWrite(LED_PIN, pressed ? HIGH : LOW);

  Serial.println(pressed ? "BUTTON: PRESSED" : "BUTTON: RELEASED");
  delay(100);
}

# 5. Expected Result

When the virtual button is pressed, LED = ON and BUTTON: PRESSED is printed. When released, LED = OFF and BUTTON: RELEASED is printed.

# 6. Troubleshooting / Verification

In Wokwi, check that the button wiring matches INPUT_PULLUP: one side to GPIO 18 and the other to GND.

If the LED logic is reversed, check LED polarity and resistor connection.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Add a second LED on GPIO 4. Make LED1 ON/LED2 OFF when pressed, and LED1 OFF/LED2 ON when released.