Practical Task 3 — ESP32 GPIO: Digital Input and Output Control

ESP32 / IoT Practical Course — Corrected Student Solution

## 1. Objective

Interface a pushbutton and LED using ESP32 digital GPIO. Demonstrate direct button-to-LED control and understand active-LOW input using the internal pull-up resistor.

## 2. Components / Software

ESP32 DevKit, breadboard, pushbutton, LED, 220 Ω resistor, jumper wires, USB cable, PlatformIO/Arduino framework.

Recommended pins: GPIO 18 for the pushbutton and GPIO 2 for the LED.

## 3. Circuit Connections

LED: GPIO 2 → 220 Ω resistor → LED anode; LED cathode → GND.

Pushbutton: one terminal → GPIO 18; other terminal → GND.

Use INPUT_PULLUP. Therefore: released = HIGH, pressed = LOW.

Do not connect an LED directly without a current-limiting resistor.

## 4. Procedure

Create/open the PlatformIO ESP32 project.

Wire the LED and pushbutton according to the circuit connections above.

Set the button pin to INPUT_PULLUP and LED pin to OUTPUT.

Read the button with digitalRead().

When the button reads LOW, turn the LED ON; otherwise turn it OFF.

Open Serial Monitor at 115200 baud and verify BUTTON and LED states.

After direct control works, implement the optional toggle extension.

## 5. Complete Program — Direct Button Control

#include <Arduino.h>

const int BUTTON_PIN = 18;
const int LED_PIN = 2;

void setup() {
  Serial.begin(115200);

  pinMode(BUTTON_PIN, INPUT_PULLUP);
  pinMode(LED_PIN, OUTPUT);

  digitalWrite(LED_PIN, LOW);
}

void loop() {
  bool pressed = (digitalRead(BUTTON_PIN) == LOW);

  digitalWrite(LED_PIN, pressed ? HIGH : LOW);

  Serial.print("BUTTON: ");
  Serial.print(pressed ? "PRESSED" : "RELEASED");
  Serial.print(" | LED: ");
  Serial.println(pressed ? "ON" : "OFF");

  delay(100);
}

## 6. Expected Result

While the pushbutton is held: BUTTON: PRESSED | LED: ON.

After the button is released: BUTTON: RELEASED | LED: OFF.

The Serial Monitor should continuously show the corresponding digital state.

## 7. Key Theory

GPIO means General-Purpose Input/Output. A GPIO can be configured as an input for reading a switch or as an output for controlling an LED.

Digital signals normally have two logic states: HIGH and LOW.

INPUT_PULLUP activates an internal pull-up resistor. With the shown wiring, the input is HIGH when the button is open and LOW when the button connects the pin to ground.

This is called active-LOW logic: the event of interest (button pressed) corresponds to LOW.

A floating input has no defined electrical state and may randomly change between HIGH and LOW. A pull-up or pull-down provides a stable default state.

## 8. Optional Toggle Extension

#include <Arduino.h>

const int BUTTON_PIN = 18;
const int LED_PIN = 2;

bool ledState = false;
bool lastReading = HIGH;
bool stableState = HIGH;
unsigned long lastChange = 0;
const unsigned long DEBOUNCE_MS = 50;

void setup() {
  Serial.begin(115200);
  pinMode(BUTTON_PIN, INPUT_PULLUP);
  pinMode(LED_PIN, OUTPUT);
}

void loop() {
  bool reading = digitalRead(BUTTON_PIN);

  if (reading != lastReading) {
    lastChange = millis();
    lastReading = reading;
  }

  if (millis() - lastChange > DEBOUNCE_MS && reading != stableState) {
    stableState = reading;

    if (stableState == LOW) {
      ledState = !ledState;
      digitalWrite(LED_PIN, ledState ? HIGH : LOW);

      Serial.print("BUTTON PRESS -> LED: ");
      Serial.println(ledState ? "ON" : "OFF");
    }
  }
}

## 9. Troubleshooting / Verification

If the LED does not light, check LED polarity and the 220 Ω resistor.

If the button always reads PRESSED, check that GPIO 18 is not permanently connected to GND.

If the button behaves randomly, verify INPUT_PULLUP is enabled and the wiring is correct.

If the logic appears reversed, remember that INPUT_PULLUP makes the pressed state LOW.

## 10. Student Evidence

Submit a clear circuit photograph or Wokwi screenshot.

Submit the PlatformIO source code.

Show Serial Monitor output for both PRESSED and RELEASED states.

Demonstrate the optional toggle behavior if assigned.