# Practical Task 4 — Pushbutton Debouncing and DHT11 Temperature-Humidity Monitoring

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Read DHT11 temperature/humidity data and use a debounced pushbutton to switch between temperature and humidity display.

# 2. Components / Software

ESP32, DHT11, pushbutton, jumper wires, breadboard. Use a 10kΩ pull-up if the DHT11 module requires an external pull-up.

# 3. Procedure

Connect DHT11 VCC to 3.3V, GND to GND, and DATA to GPIO 4.

Connect the pushbutton between GPIO 18 and GND; configure INPUT_PULLUP.

Install the DHT sensor library and Adafruit Unified Sensor library in PlatformIO.

Read the DHT11 approximately once every 2 seconds.

Debounce the button in software using a stable-state timer.

Each valid press alternates the displayed value between temperature and humidity.

Verify the sensor values in Serial Monitor.

# 4. Solution / Program

Install the required libraries in PlatformIO before compiling. Then use the following src/main.cpp:

#include <Arduino.h>
#include <DHT.h>

#define DHTPIN 4
#define DHTTYPE DHT11
const int BUTTON_PIN = 18;

DHT dht(DHTPIN, DHTTYPE);

bool lastRaw = HIGH;
bool stableState = HIGH;
unsigned long lastChange = 0;
const unsigned long DEBOUNCE_MS = 50;

bool showTemperature = true;
float temperature = NAN;
float humidity = NAN;
unsigned long lastRead = 0;

void setup() {
  Serial.begin(115200);
  pinMode(BUTTON_PIN, INPUT_PULLUP);
  dht.begin();
}

void readSensor() {
  float h = dht.readHumidity();
  float t = dht.readTemperature();

  if (!isnan(h) && !isnan(t)) {
    humidity = h;
    temperature = t;
  } else {
    Serial.println("DHT11 read failed");
  }
}

void loop() {
  if (millis() - lastRead >= 2000) {
    lastRead = millis();
    readSensor();

    Serial.print("Temperature: ");
    Serial.print(temperature, 1);
    Serial.print(" C | Humidity: ");
    Serial.print(humidity, 1);
    Serial.println(" %");
  }

  bool raw = digitalRead(BUTTON_PIN);

  if (raw != lastRaw) {
    lastChange = millis();
    lastRaw = raw;
  }

  if ((millis() - lastChange) > DEBOUNCE_MS && raw != stableState) {
    stableState = raw;

    if (stableState == LOW) {
      showTemperature = !showTemperature;

      if (showTemperature) {
        Serial.print("DISPLAY: Temperature = ");
        Serial.print(temperature, 1);
        Serial.println(" C");
      } else {
        Serial.print("DISPLAY: Humidity = ");
        Serial.print(humidity, 1);
        Serial.println(" %");
      }
    }
  }
}

# 5. Expected Result

A valid button press changes the selected display between Temperature and Humidity. DHT11 values are updated periodically without using the button as an unfiltered input.

# 6. Troubleshooting / Verification

DHT11 reads should not be requested too rapidly; the solution uses a 2-second interval.

If readings are NaN, check DATA/VCC/GND wiring and the DHT11 library installation.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Add LED on GPIO 2: normal temperature = LED OFF; temperature at/above a selected threshold = LED ON. Optionally add the buzzer.