# Practical Task 5 — I²C Communication and OLED Sensor Display

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Use I²C communication to display DHT11 environmental values on a 0.96-inch SSD1306 OLED and switch between display screens.

# 2. Components / Software

ESP32, SSD1306 I²C OLED, DHT11, pushbutton, jumper wires.

# 3. Procedure

Connect OLED VCC to 3.3V and GND to GND.

Connect OLED SDA to GPIO 21 and SCL to GPIO 22 (common ESP32 default I²C pins).

Connect DHT11 DATA to GPIO 4.

Install Adafruit SSD1306, Adafruit GFX, DHT sensor, and Adafruit Unified Sensor libraries.

Initialize the OLED at I²C address 0x3C. If the module uses another address, scan the I²C bus or change the address.

Use the button on GPIO 18 to switch between temperature and humidity screens.

# 4. Solution / Program

Install the required libraries in PlatformIO before compiling. Then use the following src/main.cpp:

#include <Arduino.h>
#include <Wire.h>
#include <Adafruit_GFX.h>
#include <Adafruit_SSD1306.h>
#include <DHT.h>

#define SCREEN_WIDTH 128
#define SCREEN_HEIGHT 64
#define OLED_ADDR 0x3C
#define DHTPIN 4
#define DHTTYPE DHT11

const int BUTTON_PIN = 18;

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);
DHT dht(DHTPIN, DHTTYPE);

bool showTemp = true;
bool lastButton = HIGH;
unsigned long lastDebounce = 0;
const unsigned long DEBOUNCE_MS = 50;

float t = 0, h = 0;
unsigned long lastRead = 0;

void showScreen() {
  display.clearDisplay();
  display.setTextColor(SSD1306_WHITE);
  display.setTextSize(1);
  display.setCursor(0, 0);
  display.println("SMART ENVIRONMENT");

  display.setTextSize(2);
  if (showTemp) {
    display.setCursor(0, 22);
    display.println("TEMP");
    display.setCursor(0, 45);
    display.print(t, 1);
    display.println(" C");
  } else {
    display.setCursor(0, 22);
    display.println("HUMIDITY");
    display.setCursor(0, 45);
    display.print(h, 1);
    display.println(" %");
  }
  display.display();
}

void setup() {
  Serial.begin(115200);
  pinMode(BUTTON_PIN, INPUT_PULLUP);
  dht.begin();

  Wire.begin(21, 22);

  if (!display.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR)) {
    Serial.println("OLED initialization failed");
    while (true) delay(1000);
  }

  display.clearDisplay();
  display.display();
}

void loop() {
  if (millis() - lastRead >= 2000) {
    lastRead = millis();
    float nt = dht.readTemperature();
    float nh = dht.readHumidity();
    if (!isnan(nt) && !isnan(nh)) {
      t = nt; h = nh;
      showScreen();
    }
  }

  bool raw = digitalRead(BUTTON_PIN);
  if (raw != lastButton) {
    lastDebounce = millis();
    lastButton = raw;
  }

  if (raw == LOW && millis() - lastDebounce > DEBOUNCE_MS) {
    showTemp = !showTemp;
    showScreen();
    while (digitalRead(BUTTON_PIN) == LOW) delay(5);
    lastButton = HIGH;
  }
}

# 5. Expected Result

OLED shows SMART ENVIRONMENT with either TEMP/value or HUMIDITY/value. Pressing the button switches screens.

# 6. Troubleshooting / Verification

Common SSD1306 I²C address is 0x3C; some displays use 0x3D.

If the OLED is blank, verify SDA/SCL, power, address, and library installation.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Create three pages: Temperature/Humidity, System Status = NORMAL, and Student Name/Roll Number.