# Practical Task 6 — PWM LED Control and Event-Driven ESP32 Programming

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Control LED brightness using PWM, generate periodic activity without long blocking delays, and handle button events using an interrupt.

# 2. Components / Software

ESP32, LED, 220Ω resistor, pushbutton, jumper wires.

# 3. Procedure

Connect LED through 220Ω to GPIO 2 and GND.

Connect pushbutton between GPIO 18 and GND using INPUT_PULLUP.

Use ESP32 PWM through analogWrite for the brightness demonstration.

Use esp_timer to generate periodic callbacks instead of a long delay loop.

Attach a GPIO interrupt to the button and increment an event counter in the ISR.

Use volatile variables in the ISR and keep the ISR short.

# 4. Solution / Program

Use the following src/main.cpp:

#include <Arduino.h>
#include "esp_timer.h"

const int LED_PIN = 2;
const int BUTTON_PIN = 18;

volatile uint32_t interruptCount = 0;
volatile bool buttonEvent = false;

esp_timer_handle_t periodicTimer;

void IRAM_ATTR buttonISR() {
  static uint32_t lastUs = 0;
  uint32_t now = micros();
  if (now - lastUs > 50000) {   // simple 50 ms ISR debounce
    interruptCount++;
    buttonEvent = true;
    lastUs = now;
  }
}

void periodicCallback(void* arg) {
  static bool state = false;
  state = !state;
  digitalWrite(LED_PIN, state);
}

void setup() {
  Serial.begin(115200);

  pinMode(LED_PIN, OUTPUT);
  pinMode(BUTTON_PIN, INPUT_PULLUP);

  // PWM brightness sequence
  Serial.println("PWM: 0,25,50,75,100 percent");
  for (int duty = 0; duty <= 255; duty += 64) {
    analogWrite(LED_PIN, duty);
    delay(500);
  }
  analogWrite(LED_PIN, 0);

  attachInterrupt(digitalPinToInterrupt(BUTTON_PIN), buttonISR, FALLING);

  esp_timer_create_args_t timerArgs = {};
  timerArgs.callback = &periodicCallback;
  timerArgs.name = "led_periodic";
  esp_timer_create(&timerArgs, &periodicTimer);
  esp_timer_start_periodic(periodicTimer, 1000000); // 1 second
}

void loop() {
  if (buttonEvent) {
    noInterrupts();
    uint32_t count = interruptCount;
    buttonEvent = false;
    interrupts();

    Serial.print("Interrupt Count: ");
    Serial.println(count);
  }

  delay(10);
}

# 5. Expected Result

The LED demonstrates increasing PWM brightness, then periodic ON/OFF activity occurs about every second. Each button press increments Interrupt Count.

# 6. Troubleshooting / Verification

Keep ISR code short. Do not perform Serial.print or other lengthy operations inside the ISR.

If interrupt counts jump unexpectedly, increase the debounce interval.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Use the button interrupt to toggle between RUNNING and PAUSED modes and display the mode/event count on the OLED.