# Practical Task 7 — LDR-Based Analog Sensing and Automatic Light Control

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Read an LDR through the ESP32 ADC, classify light level as DARK/NORMAL/BRIGHT, and control LED output automatically.

# 2. Components / Software

ESP32, LDR, 10kΩ resistor, LED, 220Ω resistor, breadboard, jumper wires.

# 3. Procedure

Build a voltage divider using the LDR and 10kΩ resistor.

Connect the divider output to an ADC-capable pin such as GPIO 34.

Connect the LED through 220Ω to GPIO 2.

Read the ADC value with analogRead.

Choose threshold values experimentally because the exact ADC reading depends on divider orientation and lighting.

Classify the environment as DARK, NORMAL, or BRIGHT.

Use the classification to implement a night lamp.

# 4. Solution / Program

Use the following src/main.cpp:

#include <Arduino.h>

const int LDR_PIN = 34;  // ADC input
const int LED_PIN = 2;

// Calibrate these values for the actual circuit.
const int DARK_THRESHOLD = 1200;
const int BRIGHT_THRESHOLD = 2800;

void setup() {
  Serial.begin(115200);
  pinMode(LED_PIN, OUTPUT);
  analogReadResolution(12); // 0..4095
}

void loop() {
  int ldr = analogRead(LDR_PIN);

  const char* level;
  if (ldr < DARK_THRESHOLD) {
    level = "DARK";
    digitalWrite(LED_PIN, HIGH);
  } else if (ldr > BRIGHT_THRESHOLD) {
    level = "BRIGHT";
    digitalWrite(LED_PIN, LOW);
  } else {
    level = "NORMAL";
    digitalWrite(LED_PIN, LOW);
  }

  Serial.print("LDR: ");
  Serial.print(ldr);
  Serial.print(" | LIGHT: ");
  Serial.println(level);

  delay(500);
}

# 5. Expected Result

Serial Monitor reports a numeric LDR reading and DARK/NORMAL/BRIGHT classification. In the selected circuit orientation, DARK turns the night-lamp LED ON and BRIGHT turns it OFF. If readings move in the opposite direction, reverse the threshold comparisons.

# 6. Troubleshooting / Verification

ADC thresholds are examples and must be calibrated for the actual LDR divider.

If DARK and BRIGHT appear reversed, reverse the comparison direction.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Use PWM so very dark = 100%, medium = 50%, and bright = 0% LED brightness. Calibrate thresholds from measured room/light readings.