# Practical Task 8 — ESP32 Wi-Fi Connectivity and HTTP Communication

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Connect the ESP32 to Wi-Fi, display its IP address, and perform an HTTP GET request.

# 2. Components / Software

ESP32, USB cable, Wi-Fi network with credentials, computer/Internet or instructor-provided HTTP test endpoint.

# 3. Procedure

Replace WIFI_SSID and WIFI_PASSWORD with the lab network credentials.

Upload the program.

Wait until Connected! appears and record the assigned IP address.

Perform an HTTP GET to the instructor-provided test endpoint. For initial testing, a simple public test endpoint may be used if permitted by the lab.

Print the HTTP response code and response body.

Do not hard-code credentials into code submitted to a public repository.

# 4. Solution / Program

Use the following src/main.cpp:

#include <Arduino.h>
#include <WiFi.h>
#include <HTTPClient.h>

const char* WIFI_SSID = "YOUR_WIFI_SSID";
const char* WIFI_PASSWORD = "YOUR_WIFI_PASSWORD";

// Replace with the instructor-provided endpoint.
const char* TEST_URL = "http://httpbin.org/get";

void setup() {
  Serial.begin(115200);
  WiFi.mode(WIFI_STA);
  WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

  Serial.print("Connecting");
  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
    Serial.print(".");
  }

  Serial.println();
  Serial.println("Connected!");
  Serial.print("IP: ");
  Serial.println(WiFi.localIP());

  HTTPClient http;
  http.begin(TEST_URL);

  int code = http.GET();
  Serial.print("HTTP Response Code: ");
  Serial.println(code);

  if (code > 0) {
    String payload = http.getString();
    Serial.println("Response:");
    Serial.println(payload);
  } else {
    Serial.println("HTTP request failed");
  }

  http.end();
}

void loop() {
  // Keep this task simple; the request is demonstrated once.
}

# 5. Expected Result

Serial Monitor should show Connecting..., Connected!, an IP address such as 192.168.x.x, an HTTP response code, and the returned response body.

# 6. Troubleshooting / Verification

If Wi-Fi does not connect, verify SSID/password and that the network permits the ESP32.

If the test endpoint is unavailable, use the instructor-provided endpoint instead.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Read DHT11 data and send temperature/humidity as HTTP query parameters to an instructor-provided server endpoint.