# Practical Task 9 — MQTT Publish and Subscribe with ESP32

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Use MQTT to publish temperature/humidity telemetry and subscribe to an LED command topic for remote control.

# 2. Components / Software

ESP32, DHT11, LED, 220Ω resistor, Wi-Fi network, MQTT broker.

# 3. Procedure

Connect DHT11 DATA to GPIO 4 and LED through 220Ω to GPIO 2.

Install PubSubClient and DHT sensor libraries.

Set a unique BASE topic using the student's roll number.

Connect the ESP32 to Wi-Fi and then to the MQTT broker.

Publish temperature and humidity periodically.

Subscribe to the LED command topic.

Send ON/OFF messages from an MQTT client and verify the ESP32 LED responds.

# 4. Solution / Program

Use the following src/main.cpp:

#include <Arduino.h>
#include <WiFi.h>
#include <PubSubClient.h>
#include <DHT.h>

const char* WIFI_SSID = "YOUR_WIFI_SSID";
const char* WIFI_PASSWORD = "YOUR_WIFI_PASSWORD";

const char* MQTT_BROKER = "broker.hivemq.com";
const int MQTT_PORT = 1883;

const char* BASE = "ntu/YOUR_ROLLNO";
String topicTemp = String(BASE) + "/temperature";
String topicHum  = String(BASE) + "/humidity";
String topicLed  = String(BASE) + "/led";

const int LED_PIN = 2;
#define DHTPIN 4
#define DHTTYPE DHT11

WiFiClient espClient;
PubSubClient mqtt(espClient);
DHT dht(DHTPIN, DHTTYPE);

unsigned long lastPublish = 0;

void callback(char* topic, byte* payload, unsigned int length) {
  String msg;
  for (unsigned int i = 0; i < length; i++) msg += (char)payload[i];

  Serial.print("Command [");
  Serial.print(topic);
  Serial.print("] = ");
  Serial.println(msg);

  if (msg.equalsIgnoreCase("ON")) digitalWrite(LED_PIN, HIGH);
  if (msg.equalsIgnoreCase("OFF")) digitalWrite(LED_PIN, LOW);
}

void connectWiFi() {
  WiFi.begin(WIFI_SSID, WIFI_PASSWORD);
  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
    Serial.print(".");
  }
  Serial.println("\nWi-Fi connected");
}

void reconnectMQTT() {
  while (!mqtt.connected()) {
    String clientId = String("ESP32-") + String((uint32_t)ESP.getEfuseMac(), HEX);
    if (mqtt.connect(clientId.c_str())) {
      Serial.println("MQTT connected");
      mqtt.subscribe(topicLed.c_str());
    } else {
      Serial.print("MQTT failed, state=");
      Serial.println(mqtt.state());
      delay(2000);
    }
  }
}

void setup() {
  Serial.begin(115200);
  pinMode(LED_PIN, OUTPUT);
  dht.begin();

  connectWiFi();
  mqtt.setServer(MQTT_BROKER, MQTT_PORT);
  mqtt.setCallback(callback);
}

void loop() {
  if (!mqtt.connected()) reconnectMQTT();
  mqtt.loop();

  if (millis() - lastPublish >= 5000) {
    lastPublish = millis();

    float t = dht.readTemperature();
    float h = dht.readHumidity();

    if (!isnan(t) && !isnan(h)) {
      mqtt.publish(topicTemp.c_str(), String(t, 1).c_str());
      mqtt.publish(topicHum.c_str(), String(h, 1).c_str());

      Serial.print("Published T=");
      Serial.print(t, 1);
      Serial.print(" H=");
      Serial.println(h, 1);
    }
  }
}

# 5. Expected Result

An MQTT client subscribed to ntu/YOUR_ROLLNO/temperature and /humidity receives telemetry. Publishing ON or OFF to ntu/YOUR_ROLLNO/led controls the ESP32 LED.

# 6. Troubleshooting / Verification

Use a unique MQTT topic containing the student's roll number.

If connection fails, check Wi-Fi, broker address/port, and that the MQTT client is using the exact same topic.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Add the LDR value as ntu/YOUR_ROLLNO/light and accept numeric 0,25,50,75,100 commands for LED brightness.