Practical Task 10 — Node-RED Fundamentals: MQTT and IoT Dashboard

ESP32 / IoT Practical Course — Corrected Student Solution

## 1. Objective

Use Node-RED as a visual IoT programming environment to receive ESP32 MQTT telemetry, inspect messages, and visualize temperature, humidity, and light data on a dashboard.

## 2. Components / Software

Computer with Node-RED installed and running.

MQTT broker access. For the course example, use broker.hivemq.com on port 1883.

ESP32 running Task 9 MQTT publish/subscribe code.

Node-RED MQTT nodes and a dashboard package available in the laboratory environment.

## 3. MQTT Data from Task 9

Temperature topic: ntu/YOUR_ROLLNO/temperature

Humidity topic: ntu/YOUR_ROLLNO/humidity

Light topic: ntu/YOUR_ROLLNO/light

LED command topic: ntu/YOUR_ROLLNO/led

Replace YOUR_ROLLNO with the student's actual roll number.

## 4. Node-RED Theory

Node-RED is a flow-based, low-code programming tool. Applications are created by connecting nodes with wires.

A flow is a visual representation of how messages move through the system.

An MQTT input node subscribes to a topic and creates a Node-RED message.

The main data is normally carried in msg.payload.

A Debug node is used to inspect incoming messages and diagnose problems.

Dashboard nodes present data to a human user as gauges, charts, text or other widgets.

Deploying a flow activates the configuration running in Node-RED.

## 5. Required Flow

ESP32
  ↓
MQTT Broker
  ↓
Node-RED MQTT IN
  ↓
Debug
  ↓
Dashboard

Create three similar paths:
temperature → Debug → dashboard
humidity    → Debug → dashboard
light       → Debug → dashboard

## 6. Procedure

Start Node-RED and open the editor, normally through the local Node-RED web interface.

Create a new flow named ESP32 IoT Dashboard.

Add an MQTT-in node for the temperature topic.

Configure the MQTT broker as broker.hivemq.com and port 1883, or use the broker specified by the instructor.

Set the topic to ntu/YOUR_ROLLNO/temperature.

Connect the MQTT-in node to a Debug node and deploy.

Run the ESP32 MQTT program and confirm that temperature messages appear in the Debug panel.

Repeat for humidity and light topics.

Add dashboard visualization nodes and connect the corresponding MQTT messages.

Deploy the complete flow and open the dashboard.

Verify that the dashboard values change when the ESP32 publishes new readings.

## 7. Node-RED Flow Construction

Temperature: MQTT IN → Debug → Temperature Gauge/Chart.

Humidity: MQTT IN → Debug → Humidity Gauge/Chart.

Light: MQTT IN → Debug → Light Gauge/Chart.

Keep Debug nodes connected during development because they make MQTT payload and topic errors much easier to identify.

## 8. Payload Handling

MQTT payloads may arrive as strings. A dashboard widget may require a numeric value.

If the widget does not accept the value correctly, insert a Change or Function node to convert msg.payload to a number.

Example Function node: msg.payload = Number(msg.payload); return msg;

## 9. Expected Result

Node-RED Debug shows MQTT messages arriving from the ESP32.

The dashboard displays live temperature, humidity and light values.

The values update as new MQTT messages are published.

## 10. Troubleshooting

If Debug receives nothing, verify Wi-Fi, MQTT broker, topic spelling and the ESP32 publish code.

If Debug receives data but the dashboard is blank, check the dashboard widget configuration.

If a gauge reports an invalid value, inspect msg.payload and convert it to Number when necessary.

Always click Deploy after modifying the flow.

Use a unique YOUR_ROLLNO topic namespace for each student to prevent students from seeing one another's data.

## 11. Extension / Home Task

Add a dashboard status indicator for temperature above a selected threshold.

Add the LDR/light value as a chart.

Prepare the flow so it can be extended in Task 11 with threshold processing, alerts and MQTT LED control.

## 12. Student Evidence

Submit the Node-RED flow or exported flow JSON.

Show the Debug panel receiving MQTT messages.

Show the dashboard with sensor values.

Demonstrate the ESP32 publishing live data.