# Practical Task 11 — Node-RED Data Processing, Alerts and IoT Automation

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Process MQTT sensor values, classify conditions, generate warnings, and send an MQTT command back to the ESP32 for automatic LED control.

# 2. Components / Software

Node-RED, MQTT broker, ESP32 MQTT program from Task 9, DHT11/LDR, LED.

# 3. Procedure

Reuse the Task 10 MQTT input flow.

Convert incoming payload strings to numbers using a Change or Function node.

Use a Switch node for temperature threshold logic, e.g. >= 30 °C = HIGH.

Use another Switch/Function node to classify LDR as DARK, NORMAL, or BRIGHT using calibrated thresholds.

Display status/warnings on the dashboard.

Create an MQTT output topic ntu/YOUR_ROLLNO/led and send ON when the automatic night-lamp condition is true and OFF otherwise.

Deploy and test with real sensor readings or controlled test messages.

# 4. Solution / Program

Node-RED solution:

Example Function node for temperature status:

const t = Number(msg.payload);
msg.payload = {
    value: t,
    status: t >= 30 ? "HIGH TEMPERATURE" : "NORMAL"
};
return msg;

Example Function node for LDR classification:

const x = Number(msg.payload);
if (x < 1200) msg.payload = "DARK";
else if (x > 2800) msg.payload = "BRIGHT";
else msg.payload = "NORMAL";
return msg;

Automatic LED control:
LDR input -> number/classification -> Switch
DARK -> MQTT OUT topic ntu/YOUR_ROLLNO/led payload ON
NORMAL/BRIGHT -> MQTT OUT topic ntu/YOUR_ROLLNO/led payload OFF

Use thresholds appropriate to the actual LDR voltage-divider orientation.

# 5. Expected Result

Node-RED identifies HIGH TEMPERATURE or NORMAL and classifies light as DARK/NORMAL/BRIGHT. When DARK is detected, an MQTT ON command is sent to the ESP32 and the LED turns ON.

# 6. Troubleshooting / Verification

Test threshold logic first with injected MQTT values such as 25, 30, 35 and LDR values around the selected thresholds.

Make sure the MQTT OUT topic exactly matches the ESP32 subscription topic.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Add a warning indicator for high temperature and a dashboard text/status node. Test edge cases around each threshold.