# Practical Task 12 — Blynk IoT Dashboard and ESP32 Remote Control

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Send DHT11/LDR telemetry from ESP32 to Blynk and receive a remote LED control command from a Blynk dashboard.

# 2. Components / Software

ESP32, DHT11, LDR, LED, 220Ω resistor, Wi-Fi, Blynk account/app/web dashboard.

# 3. Procedure

Create a Blynk template/device and obtain the Template ID, Template Name, and Auth Token.

Create datastreams/virtual pins: V0 Temperature, V1 Humidity, V2 Light, V3 LED control.

Place widgets such as temperature/humidity gauges, a light value widget, and a switch button.

Install the Blynk library in PlatformIO.

Replace the placeholders in the code with the student's Blynk credentials and Wi-Fi credentials.

Upload the ESP32 program.

Verify sensor data appears in the dashboard.

Use the Blynk switch to control the ESP32 LED.

# 4. Solution / Program

Blynk setup: create the template/device first and replace all YOUR_* placeholders. Then use the following src/main.cpp:

#define BLYNK_TEMPLATE_ID "YOUR_TEMPLATE_ID"
#define BLYNK_TEMPLATE_NAME "YOUR_TEMPLATE_NAME"
#define BLYNK_AUTH_TOKEN "YOUR_AUTH_TOKEN"

#include <Arduino.h>
#include <WiFi.h>
#include <BlynkSimpleEsp32.h>
#include <DHT.h>

char ssid[] = "YOUR_WIFI_SSID";
char pass[] = "YOUR_WIFI_PASSWORD";

#define DHTPIN 4
#define DHTTYPE DHT11

const int LDR_PIN = 34;
const int LED_PIN = 2;

DHT dht(DHTPIN, DHTTYPE);
BlynkTimer timer;

void sendSensorData() {
  float t = dht.readTemperature();
  float h = dht.readHumidity();
  int light = analogRead(LDR_PIN);

  if (!isnan(t)) Blynk.virtualWrite(V0, t);
  if (!isnan(h)) Blynk.virtualWrite(V1, h);
  Blynk.virtualWrite(V2, light);
}

BLYNK_WRITE(V3) {
  int value = param.asInt();
  digitalWrite(LED_PIN, value ? HIGH : LOW);
}

void setup() {
  Serial.begin(115200);
  pinMode(LED_PIN, OUTPUT);
  analogReadResolution(12);
  dht.begin();

  Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);
  timer.setInterval(2000L, sendSensorData);
}

void loop() {
  Blynk.run();
  timer.run();
}

# 5. Expected Result

Blynk displays temperature, humidity, and light values. Pressing the V3 switch changes the ESP32 LED state remotely.

# 6. Troubleshooting / Verification

If Blynk stays offline, verify Template ID, Template Name, Auth Token, Wi-Fi credentials, and device/template configuration.

Check that widget datastreams use the same virtual pins as the code.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Add a Blynk slider for LED brightness and implement PWM output. Keep the sensor datastreams and control datastream separate.