# Practical Task 13 — Integrated Multi-Sensor IoT Monitoring and Control System

ESP32 / IoT Practical Course — Student Solution

# 1. Objective

Integrate DHT11, LDR, OLED, Wi-Fi, MQTT, Node-RED/Blynk, and automatic/remote LED control into one coherent IoT system.

# 2. Components / Software

ESP32, DHT11, LDR + 10kΩ resistor, SSD1306 OLED, LED + 220Ω resistor, pushbutton, breadboard, Wi-Fi, MQTT broker, Node-RED and/or Blynk.

# 3. Procedure

Wire DHT11 DATA to GPIO 4, LDR divider output to GPIO 34, OLED SDA/SCL to GPIO 21/22, LED to GPIO 2, and optional button to GPIO 18.

Read DHT11 and LDR values at suitable intervals.

Display temperature, humidity, light level, and system status on the OLED.

Connect the ESP32 to Wi-Fi.

Publish temperature, humidity, and light data using MQTT with student-specific topics.

Use Node-RED to process/visualize the MQTT data and optionally use Blynk for an interactive remote dashboard.

Implement remote LED control through MQTT and/or Blynk.

Implement an automatic rule: when LDR indicates DARK, turn the LED ON and show the status on the OLED.

Test sensor failure, Wi-Fi reconnection, MQTT reconnection, and threshold behavior.

# 4. Solution / Program

Use the following src/main.cpp:

#include <Arduino.h>
#include <Wire.h>
#include <WiFi.h>
#include <PubSubClient.h>
#include <Adafruit_GFX.h>
#include <Adafruit_SSD1306.h>
#include <DHT.h>

const char* WIFI_SSID = "YOUR_WIFI_SSID";
const char* WIFI_PASSWORD = "YOUR_WIFI_PASSWORD";
const char* MQTT_BROKER = "broker.hivemq.com";
const int MQTT_PORT = 1883;

const char* BASE = "ntu/YOUR_ROLLNO";
String tTopic = String(BASE) + "/temperature";
String hTopic = String(BASE) + "/humidity";
String lTopic = String(BASE) + "/light";
String ledTopic = String(BASE) + "/led";

#define DHTPIN 4
#define DHTTYPE DHT11
const int LDR_PIN = 34;
const int LED_PIN = 2;

Adafruit_SSD1306 oled(128, 64, &Wire, -1);
DHT dht(DHTPIN, DHTTYPE);
WiFiClient net;
PubSubClient mqtt(net);

const int DARK_THRESHOLD = 1200;
float t = NAN, h = NAN;
int lightValue = 0;
unsigned long lastSensor = 0;
unsigned long lastPublish = 0;

void showOLED(const char* status) {
  oled.clearDisplay();
  oled.setTextColor(SSD1306_WHITE);
  oled.setTextSize(1);
  oled.setCursor(0, 0);
  oled.println("SMART ENVIRONMENT");
  oled.print("T: "); oled.print(t, 1); oled.println(" C");
  oled.print("H: "); oled.print(h, 1); oled.println(" %");
  oled.print("L: "); oled.println(lightValue);
  oled.print("Status: "); oled.println(status);
  oled.display();
}

void mqttCallback(char* topic, byte* payload, unsigned int length) {
  String msg;
  for (unsigned int i = 0; i < length; i++) msg += (char)payload[i];

  if (msg.equalsIgnoreCase("ON")) digitalWrite(LED_PIN, HIGH);
  if (msg.equalsIgnoreCase("OFF")) digitalWrite(LED_PIN, LOW);
}

void connectWiFi() {
  if (WiFi.status() == WL_CONNECTED) return;
  WiFi.begin(WIFI_SSID, WIFI_PASSWORD);
  Serial.print("Connecting Wi-Fi");
  unsigned long start = millis();
  while (WiFi.status() != WL_CONNECTED && millis() - start < 15000) {
    delay(300);
    Serial.print(".");
  }
  Serial.println();
}

void connectMQTT() {
  if (mqtt.connected()) return;
  String id = String("ESP32-INTEGRATED-") + String((uint32_t)ESP.getEfuseMac(), HEX);
  if (mqtt.connect(id.c_str())) {
    mqtt.subscribe(ledTopic.c_str());
    Serial.println("MQTT connected");
  }
}

void setup() {
  Serial.begin(115200);
  pinMode(LED_PIN, OUTPUT);
  analogReadResolution(12);
  dht.begin();

  Wire.begin(21, 22);
  oled.begin(SSD1306_SWITCHCAPVCC, 0x3C);
  oled.clearDisplay();
  oled.display();

  connectWiFi();
  mqtt.setServer(MQTT_BROKER, MQTT_PORT);
  mqtt.setCallback(mqttCallback);
}

void loop() {
  if (WiFi.status() != WL_CONNECTED) connectWiFi();
  if (WiFi.status() == WL_CONNECTED) {
    if (!mqtt.connected()) connectMQTT();
    mqtt.loop();
  }

  if (millis() - lastSensor >= 2000) {
    lastSensor = millis();

    float nt = dht.readTemperature();
    float nh = dht.readHumidity();
    if (!isnan(nt) && !isnan(nh)) {
      t = nt;
      h = nh;
    }

    lightValue = analogRead(LDR_PIN);

    bool dark = lightValue < DARK_THRESHOLD;
    digitalWrite(LED_PIN, dark ? HIGH : LOW);

    showOLED(dark ? "DARK - LED ON" : "NORMAL");

    Serial.print("T="); Serial.print(t, 1);
    Serial.print(" H="); Serial.print(h, 1);
    Serial.print(" L="); Serial.println(lightValue);
  }

  if (mqtt.connected() && millis() - lastPublish >= 5000) {
    lastPublish = millis();

    if (!isnan(t)) mqtt.publish(tTopic.c_str(), String(t, 1).c_str());
    if (!isnan(h)) mqtt.publish(hTopic.c_str(), String(h, 1).c_str());
    mqtt.publish(lTopic.c_str(), String(lightValue).c_str());
  }
}

# 5. Expected Result

Minimum demonstration: DHT11 and LDR readings are acquired; values appear on OLED; ESP32 connects to Wi-Fi; MQTT telemetry reaches Node-RED; Node-RED can visualize/process data; and a DARK condition automatically turns the LED ON. Remote MQTT LED control should also work.

# 6. Troubleshooting / Verification

Build the system incrementally: sensors -> OLED -> Wi-Fi -> MQTT -> Node-RED/Blynk -> control.

For final testing, verify both automatic DARK control and remote control separately before combining them.

# 7. Student Submission / Evidence

Submit: circuit/simulation evidence, source code or Node-RED flow, relevant Serial Monitor/dashboard output, and a short demonstration. Replace all YOUR_* placeholders before submission.

# 8. Extension / Home Task

Add Blynk as the interactive dashboard, or add a Node-RED warning/automation flow. The final system should clearly distinguish local OLED monitoring, MQTT telemetry, dashboard visualization, and automatic/remote control.